P9E MICROSYSTEMS PVT LTD

Chetana Space Technologies Division

# Software Requirements

# Specification (SRS)

Space Technology Platform

Requirements Engineering - Phase 3, Deliverable D3.1

| Document ID: | P9E-SPACE-SRS-D3.1-001 |
| --- | --- |
| Version: | 1.0 |
| Date: | December 27, 2025 |
| Total Requirements: | Part 1: Ground Station (33) + Platform (14) = 47 requirements |

# Table of Contents

# 1. Introduction

## 1.1 Purpose

This Software Requirements Specification (SRS) defines the functional and non-functional requirements for the P9e Space Technology Platform. Requirements are derived from the Use Case Specifications (D2.3) with full traceability.

## 1.2 Scope

This document covers requirements for all five platform modules: Ground Station Software, Earth Observation Analytics, Small Satellite Subsystems, Geospatial Intelligence, and Platform/Administration.

## 1.3 Requirements Naming Convention

| Prefix | Module | Example |
| --- | --- | --- |
| FR-GS-NNN | Ground Station | FR-GS-001 Pass Prediction Engine |
| FR-EO-NNN | Earth Observation | FR-EO-010 Catalog Search |
| FR-SAT-NNN | Satellite Subsystems | FR-SAT-001 Attitude Estimation |
| FR-GI-NNN | GEOINT | FR-GI-001 AOI Management |
| FR-PLT-NNN | Platform | FR-PLT-001 Authentication |
| NFR-NNN | Non-Functional | NFR-001 System Availability |

## 1.4 Requirement Template

Each requirement includes:

- Requirement ID: Unique identifier for traceability

- Title: Short descriptive name

- Description: 'The system shall...' statement

- Rationale: Why this requirement exists

- Source Use Case(s): Traceability to UC

- Priority: Critical/High/Medium/Low

- Acceptance Criteria: Testable verification conditions

- Dependencies: Other requirements this depends on

- Risk: Implementation risk level

# 2. Requirements Summary

| Module | This Doc | Part 2 | Total |
| --- | --- | --- | --- |
| Ground Station Software | 33 | - | 33 |
| Earth Observation Analytics | - | 35 | 35 |
| Small Satellite Subsystems | - | 30 | 30 |
| Geospatial Intelligence | - | 25 | 25 |
| Platform / Administration | 14 | - | 14 |
| Non-Functional Requirements | - | 20 | 20 |
| TOTAL | 47 | 110 | 157 |

# 3. Ground Station Software Requirements

This section specifies 33 functional requirements for the Ground Station Software module, derived from 23 use cases.

## 3.1 Pass Scheduling Requirements (FR-GS-001 to FR-GS-005)

### FR-GS-001: Pass Prediction Engine

| Requirement ID | FR-GS-001 |
| --- | --- |
| Title | Pass Prediction Engine |
| Description | The system shall predict satellite pass windows over configured ground stations using SGP4/SDP4 propagation algorithms with TLE input data. |
| Rationale | Accurate pass prediction is essential for mission planning and resource allocation. SGP4/SDP4 are industry standard algorithms. |
| Source Use Case(s) | UC-GS-020 (Predict Pass Windows) |
| Priority | High |
| Acceptance Criteria | AC1: Predict passes with timing accuracy <1 second at 24-hour horizon. AC2: Support prediction for 1000+ satellites. AC3: Complete prediction for all satellites in <5 minutes. |
| Dependencies | FR-GS-010 (TLE Database) |
| Risk | Medium - Algorithm accuracy depends on TLE freshness |

### FR-GS-002: Pass Scheduling Interface

| Requirement ID | FR-GS-002 |
| --- | --- |
| Title | Pass Scheduling Interface |
| Description | The system shall provide a graphical interface for operators to view predicted passes and schedule satellite contacts on a timeline view. |
| Rationale | Visual timeline enables efficient scheduling decisions and conflict identification. |
| Source Use Case(s) | UC-GS-001 (Schedule Satellite Pass) |
| Priority | High |
| Acceptance Criteria | AC1: Display passes in Gantt-style timeline. AC2: Support drag-and-drop scheduling. AC3: Show resource conflicts visually. AC4: Filter by satellite, station, date range. |
| Dependencies | FR-GS-001 (Pass Prediction) |
| Risk | Low |

### FR-GS-003: Resource Reservation

| Requirement ID | FR-GS-003 |
| --- | --- |
| Title | Resource Reservation |
| Description | The system shall reserve ground station resources (antenna, personnel, RF equipment) when a pass is scheduled and release them upon completion or cancellation. |
| Rationale | Prevents double-booking and ensures resources are available when needed. |
| Source Use Case(s) | UC-GS-001 (Schedule Satellite Pass) |
| Priority | High |
| Acceptance Criteria | AC1: Prevent conflicting reservations. AC2: Send confirmation upon successful reservation. AC3: Support reservation modification. AC4: Auto-release after pass window. |
| Dependencies | FR-GS-002 (Scheduling Interface) |
| Risk | Low |

### FR-GS-004: Schedule Conflict Detection

| Requirement ID | FR-GS-004 |
| --- | --- |
| Title | Schedule Conflict Detection |
| Description | The system shall automatically detect scheduling conflicts when multiple passes compete for the same resources and suggest resolution options. |
| Rationale | Proactive conflict detection prevents operational issues during pass execution. |
| Source Use Case(s) | UC-GS-021 (Resolve Schedule Conflicts) |
| Priority | High |
| Acceptance Criteria | AC1: Detect conflicts in real-time during scheduling. AC2: Suggest alternative windows based on priority. AC3: Allow manual override with confirmation. AC4: Log all conflict resolutions. |
| Dependencies | FR-GS-003 (Resource Reservation) |
| Risk | Low |

### FR-GS-005: Pass Notification System

| Requirement ID | FR-GS-005 |
| --- | --- |
| Title | Pass Notification System |
| Description | The system shall send notifications to assigned personnel at configurable intervals before scheduled passes (e.g., T-60min, T-10min, T-5min). |
| Rationale | Ensures operators are prepared and present for scheduled passes. |
| Source Use Case(s) | UC-GS-001 (Schedule Satellite Pass), UC-GS-002 (Execute Pass) |
| Priority | Medium |
| Acceptance Criteria | AC1: Support email, SMS, Slack, push notifications. AC2: Configurable notification times per mission. AC3: Include pass details in notification. AC4: Track notification delivery status. |
| Dependencies | FR-PLT-020 (Notification Service) |
| Risk | Low |

## 3.2 Pass Execution Requirements (FR-GS-010 to FR-GS-019)

### FR-GS-010: TLE Database Management

| Requirement ID | FR-GS-010 |
| --- | --- |
| Title | TLE Database Management |
| Description | The system shall maintain a database of Two-Line Element sets for all tracked satellites, with automatic updates from Space-Track.org and manual upload capability. |
| Rationale | Current TLEs are essential for accurate pass prediction and antenna pointing. |
| Source Use Case(s) | UC-GS-030 (Update TLE Database) |
| Priority | High |
| Acceptance Criteria | AC1: Auto-update from Space-Track every 6 hours. AC2: Support 10,000+ objects. AC3: Flag TLEs older than 7 days. AC4: Retain TLE history. AC5: Support manual TLE upload. |
| Dependencies | External: Space-Track.org API |
| Risk | Medium - External dependency on Space-Track availability |

### FR-GS-011: Antenna Pointing Control

| Requirement ID | FR-GS-011 |
| --- | --- |
| Title | Antenna Pointing Control |
| Description | The system shall control antenna pointing using program track (ephemeris-based) and autotrack (signal-based) modes with seamless mode transitions. |
| Rationale | Accurate antenna pointing is critical for maintaining RF link during passes. |
| Source Use Case(s) | UC-GS-013 (Control Antenna Pointing) |
| Priority | Critical |
| Acceptance Criteria | AC1: Pointing accuracy <0.1° RMS in autotrack. AC2: Update rate minimum 10 Hz. AC3: Mode transition <1 second. AC4: Support emergency stow command. AC5: Log all pointing data. |
| Dependencies | FR-GS-001 (Pass Prediction), External: Antenna Control System |
| Risk | High - Hardware interface complexity |

### FR-GS-012: Pre-Pass Checklist Automation

| Requirement ID | FR-GS-012 |
| --- | --- |
| Title | Pre-Pass Checklist Automation |
| Description | The system shall execute automated pre-pass checklist at T-5 minutes including antenna positioning, RF equipment verification, and system health checks. |
| Rationale | Automated pre-pass checks ensure system readiness and reduce operator workload. |
| Source Use Case(s) | UC-GS-002 (Execute Satellite Pass) |
| Priority | High |
| Acceptance Criteria | AC1: Execute checklist items in sequence. AC2: Report pass/fail for each item. AC3: Block pass execution if critical items fail. AC4: Allow operator override with confirmation. |
| Dependencies | FR-GS-011 (Antenna Control), FR-GS-020 (RF Configuration) |
| Risk | Medium |

### FR-GS-013: Signal Acquisition and Tracking

| Requirement ID | FR-GS-013 |
| --- | --- |
| Title | Signal Acquisition and Tracking |
| Description | The system shall acquire satellite signal at AOS using SDR, achieve carrier and frame lock, and maintain tracking throughout the pass. |
| Rationale | Signal acquisition is the foundation of all pass operations. |
| Source Use Case(s) | UC-GS-002 (Execute Satellite Pass), UC-GS-014 (Monitor Signal Quality) |
| Priority | Critical |
| Acceptance Criteria | AC1: Acquire signal within 30 seconds of predicted AOS. AC2: Display real-time C/N0, BER, AGC levels. AC3: Support automatic frequency tracking (Doppler). AC4: Alert on signal degradation. |
| Dependencies | FR-GS-011 (Antenna Control), External: SDR |
| Risk | High - RF environment variability |

### FR-GS-014: Telemetry Reception and Decommutation

| Requirement ID | FR-GS-014 |
| --- | --- |
| Title | Telemetry Reception and Decommutation |
| Description | The system shall receive CCSDS telemetry packets, decommutate parameters according to spacecraft database, and apply calibrations. |
| Rationale | Telemetry decommutation converts raw data to engineering values for monitoring. |
| Source Use Case(s) | UC-GS-003 (Monitor Real-Time Telemetry) |
| Priority | Critical |
| Acceptance Criteria | AC1: Support CCSDS packet telemetry. AC2: Decommutate 5000+ parameters per spacecraft. AC3: Apply polynomial and lookup calibrations. AC4: Handle packet errors gracefully. AC5: Latency <500ms from reception to display. |
| Dependencies | FR-GS-013 (Signal Acquisition) |
| Risk | Medium |

### FR-GS-015: Real-Time Telemetry Display

| Requirement ID | FR-GS-015 |
| --- | --- |
| Title | Real-Time Telemetry Display |
| Description | The system shall display telemetry parameters in real-time with configurable layouts, limit checking, and visual status indicators. |
| Rationale | Real-time display enables operators to monitor spacecraft health during passes. |
| Source Use Case(s) | UC-GS-003 (Monitor Real-Time Telemetry) |
| Priority | Critical |
| Acceptance Criteria | AC1: Update rate minimum 1 Hz. AC2: Support numeric, gauge, strip chart displays. AC3: Color-code by limit status (green/yellow/red). AC4: Support 100+ concurrent displays. AC5: Save/load display layouts. |
| Dependencies | FR-GS-014 (Telemetry Decommutation) |
| Risk | Low |

### FR-GS-016: Telemetry Limit Monitoring

| Requirement ID | FR-GS-016 |
| --- | --- |
| Title | Telemetry Limit Monitoring |
| Description | The system shall check telemetry parameters against configurable warning (yellow) and alarm (red) limits and generate alerts for violations. |
| Rationale | Limit monitoring enables early detection of spacecraft anomalies. |
| Source Use Case(s) | UC-GS-003 (Monitor Telemetry), UC-GS-040 (Configure Alert Thresholds) |
| Priority | Critical |
| Acceptance Criteria | AC1: Check limits within 100ms of parameter update. AC2: Support per-parameter warning and alarm limits. AC3: Support hysteresis to prevent flapping. AC4: Generate audible and visual alerts. AC5: Log all limit violations. |
| Dependencies | FR-GS-014 (Telemetry Decommutation) |
| Risk | Low |

### FR-GS-017: Telecommand Encoding and Uplink

| Requirement ID | FR-GS-017 |
| --- | --- |
| Title | Telecommand Encoding and Uplink |
| Description | The system shall encode telecommands per CCSDS standard, transmit via SDR uplink, and verify spacecraft acknowledgment. |
| Rationale | Command capability is essential for spacecraft control. |
| Source Use Case(s) | UC-GS-004 (Send Telecommand) |
| Priority | Critical |
| Acceptance Criteria | AC1: Support CCSDS telecommand packet format. AC2: Command latency <1 second to uplink. AC3: Verify command acknowledgment from telemetry. AC4: Support command retries. AC5: Complete audit trail for all commands. |
| Dependencies | FR-GS-013 (Signal Acquisition), External: SDR |
| Risk | High - Command execution has mission impact |

### FR-GS-018: Two-Person Command Authorization

| Requirement ID | FR-GS-018 |
| --- | --- |
| Title | Two-Person Command Authorization |
| Description | The system shall require two-person authorization for critical commands (e.g., propulsion, deployments, safe mode) before transmission. |
| Rationale | Critical commands have irreversible consequences and require verification. |
| Source Use Case(s) | UC-GS-004 (Send Telecommand) |
| Priority | High |
| Acceptance Criteria | AC1: Configurable list of commands requiring two-person auth. AC2: Second operator must be different user. AC3: Authorization timeout (configurable, default 60s). AC4: Log both authorizers. |
| Dependencies | FR-GS-017 (Telecommand Uplink), FR-PLT-001 (Authentication) |
| Risk | Low |

### FR-GS-019: Command Sequence Execution

| Requirement ID | FR-GS-019 |
| --- | --- |
| Title | Command Sequence Execution |
| Description | The system shall support execution of pre-defined command sequences with configurable timing and conditional execution based on telemetry. |
| Rationale | Command sequences automate routine operations and reduce operator error. |
| Source Use Case(s) | UC-GS-004 (Send Telecommand) |
| Priority | High |
| Acceptance Criteria | AC1: Support sequence definition with relative timing. AC2: Support conditional commands (wait for telemetry condition). AC3: Allow sequence pause/resume/abort. AC4: Log sequence execution with timing. |
| Dependencies | FR-GS-017 (Telecommand Uplink) |
| Risk | Medium |

## 3.3 RF and Antenna Requirements (FR-GS-020 to FR-GS-022)

### FR-GS-020: RF Equipment Configuration

| Requirement ID | FR-GS-020 |
| --- | --- |
| Title | RF Equipment Configuration |
| Description | The system shall configure SDR and RF equipment parameters including frequency, modulation, data rate, and FEC settings per mission profile. |
| Rationale | Different satellites require different RF configurations. |
| Source Use Case(s) | UC-GS-010 (Configure RF Equipment) |
| Priority | High |
| Acceptance Criteria | AC1: Store configurations as mission profiles. AC2: Push configuration to equipment in <30 seconds. AC3: Verify configuration with equipment readback. AC4: Support profile comparison and copying. |
| Dependencies | External: SDR, RF Equipment |
| Risk | Medium - Equipment-specific interfaces |

### FR-GS-021: Antenna Calibration Support

| Requirement ID | FR-GS-021 |
| --- | --- |
| Title | Antenna Calibration Support |
| Description | The system shall support antenna pointing calibration using celestial sources (stars, sun, geostationary satellites) and calculate pointing model corrections. |
| Rationale | Regular calibration maintains pointing accuracy. |
| Source Use Case(s) | UC-GS-011 (Calibrate Antenna) |
| Priority | Medium |
| Acceptance Criteria | AC1: Support multi-point calibration across sky. AC2: Calculate pointing model coefficients. AC3: Generate calibration report with residuals. AC4: Apply corrections automatically. |
| Dependencies | FR-GS-011 (Antenna Control) |
| Risk | Low |

### FR-GS-022: Station Diagnostics

| Requirement ID | FR-GS-022 |
| --- | --- |
| Title | Station Diagnostics |
| Description | The system shall perform automated diagnostic tests on ground station equipment including loopback tests, timing verification, and connectivity checks. |
| Rationale | Diagnostics ensure equipment readiness before passes. |
| Source Use Case(s) | UC-GS-012 (Run Station Diagnostics) |
| Priority | Medium |
| Acceptance Criteria | AC1: Complete full diagnostics in <15 minutes. AC2: Report pass/fail with details for each test. AC3: Store results for trending. AC4: Block operations if critical tests fail. |
| Dependencies | All ground station hardware |
| Risk | Low |

## 3.4 Data Management Requirements (FR-GS-030 to FR-GS-032)

### FR-GS-030: Pass Data Recording

| Requirement ID | FR-GS-030 |
| --- | --- |
| Title | Pass Data Recording |
| Description | The system shall record all data during satellite passes including raw RF (optional), decommutated telemetry, commands, and events with precise timestamps. |
| Rationale | Complete recording enables post-pass analysis and anomaly investigation. |
| Source Use Case(s) | UC-GS-006 (Record Pass Data) |
| Priority | Critical |
| Acceptance Criteria | AC1: Zero data loss during recording. AC2: Support 100 Mbps sustained write rate. AC3: Generate recording manifest with checksums. AC4: Support playback at variable speeds. |
| Dependencies | FR-GS-014 (Telemetry), External: Storage System |
| Risk | Medium - High data rates require robust storage |

### FR-GS-031: Telemetry Archive

| Requirement ID | FR-GS-031 |
| --- | --- |
| Title | Telemetry Archive |
| Description | The system shall archive telemetry data to long-term storage with configurable retention policies and support retrieval for historical analysis. |
| Rationale | Historical telemetry is essential for trending and anomaly investigation. |
| Source Use Case(s) | UC-GS-031 (Archive Telemetry), UC-GS-032 (Retrieve Historical) |
| Priority | High |
| Acceptance Criteria | AC1: Archive within 1 hour of pass completion. AC2: Support 10+ year retention. AC3: Retrieval time <10 seconds for hot tier. AC4: Support query by time range and parameters. |
| Dependencies | FR-GS-030 (Pass Recording), External: Object Storage |
| Risk | Low |

### FR-GS-032: Pass Report Generation

| Requirement ID | FR-GS-032 |
| --- | --- |
| Title | Pass Report Generation |
| Description | The system shall automatically generate pass summary reports including link statistics, telemetry summary, command log, and anomalies. |
| Rationale | Pass reports document operations for review and customer delivery. |
| Source Use Case(s) | UC-GS-007 (Generate Pass Report) |
| Priority | Medium |
| Acceptance Criteria | AC1: Generate report within 30 minutes of LOS. AC2: Support PDF and HTML formats. AC3: Include configurable sections. AC4: Auto-distribute to stakeholders. |
| Dependencies | FR-GS-030 (Pass Recording) |
| Risk | Low |

## 3.5 Anomaly Handling Requirements (FR-GS-040 to FR-GS-043)

### FR-GS-040: Anomaly Detection

| Requirement ID | FR-GS-040 |
| --- | --- |
| Title | Anomaly Detection |
| Description | The system shall automatically detect anomaly conditions including telemetry limit violations, equipment faults, and link degradation. |
| Rationale | Early anomaly detection enables rapid response. |
| Source Use Case(s) | UC-GS-005 (Handle Pass Anomaly), UC-GS-041 (Receive Anomaly Alert) |
| Priority | Critical |
| Acceptance Criteria | AC1: Detect anomalies within 30 seconds of occurrence. AC2: Classify severity (Critical/Major/Minor). AC3: Support configurable detection rules. AC4: Minimize false positives. |
| Dependencies | FR-GS-016 (Limit Monitoring) |
| Risk | Medium |

### FR-GS-041: Anomaly Alerting

| Requirement ID | FR-GS-041 |
| --- | --- |
| Title | Anomaly Alerting |
| Description | The system shall deliver anomaly alerts to appropriate personnel via multiple channels based on severity and escalation matrix. |
| Rationale | Timely alerts ensure rapid response to anomalies. |
| Source Use Case(s) | UC-GS-041 (Receive Anomaly Alert), UC-GS-042 (Acknowledge Alert) |
| Priority | Critical |
| Acceptance Criteria | AC1: Alert delivery <30 seconds. AC2: Support email, SMS, Slack, voice call. AC3: Escalate if not acknowledged within timeout. AC4: Track acknowledgment status. AC5: 99.99% delivery reliability. |
| Dependencies | FR-GS-040 (Anomaly Detection), FR-PLT-020 (Notification Service) |
| Risk | Low |

### FR-GS-042: Anomaly Procedure Access

| Requirement ID | FR-GS-042 |
| --- | --- |
| Title | Anomaly Procedure Access |
| Description | The system shall provide quick access to anomaly response procedures with step-by-step checklists and documentation capability. |
| Rationale | Procedures guide operators through anomaly resolution. |
| Source Use Case(s) | UC-GS-005 (Handle Pass Anomaly) |
| Priority | High |
| Acceptance Criteria | AC1: Procedure access <5 seconds. AC2: Support procedure checklists with tracking. AC3: Link procedures to anomaly types. AC4: Capture deviations from procedure. |
| Dependencies | FR-GS-040 (Anomaly Detection) |
| Risk | Low |

### FR-GS-043: Anomaly Documentation

| Requirement ID | FR-GS-043 |
| --- | --- |
| Title | Anomaly Documentation |
| Description | The system shall capture complete anomaly documentation including timeline, actions taken, telemetry snapshots, and resolution. |
| Rationale | Documentation supports post-event analysis and lessons learned. |
| Source Use Case(s) | UC-GS-005 (Handle Pass Anomaly) |
| Priority | High |
| Acceptance Criteria | AC1: Auto-capture telemetry during anomaly. AC2: Support text and voice notes. AC3: Generate anomaly report. AC4: Link to related passes and events. |
| Dependencies | FR-GS-040 (Anomaly Detection), FR-GS-030 (Pass Recording) |
| Risk | Low |

## 3.6 Multi-Station Requirements (FR-GS-050 to FR-GS-051)

### FR-GS-050: Multi-Station Coordination

| Requirement ID | FR-GS-050 |
| --- | --- |
| Title | Multi-Station Coordination |
| Description | The system shall coordinate pass handovers between multiple ground stations to extend coverage duration. |
| Rationale | Multi-station operations maximize contact time with satellites. |
| Source Use Case(s) | UC-GS-022 (Coordinate Multi-Station Pass) |
| Priority | Medium |
| Acceptance Criteria | AC1: Calculate optimal handover timing. AC2: Coordinate with remote stations via API. AC3: Handover latency <5 seconds. AC4: Zero data loss during handover. |
| Dependencies | FR-GS-001 (Pass Prediction), External: Partner Stations |
| Risk | High - Inter-station coordination complexity |

### FR-GS-051: AWS Ground Station Integration

| Requirement ID | FR-GS-051 |
| --- | --- |
| Title | AWS Ground Station Integration |
| Description | The system shall integrate with AWS Ground Station service for scheduling and data delivery using AWS APIs. |
| Rationale | AWS GS extends ground station network without capital investment. |
| Source Use Case(s) | UC-GS-022 (Coordinate Multi-Station), UC-GS-023 (Integrate External GS) |
| Priority | Medium |
| Acceptance Criteria | AC1: Schedule passes via AWS API. AC2: Receive data delivery to S3. AC3: Process data from AWS passes like local passes. AC4: Track AWS usage for billing. |
| Dependencies | External: AWS Ground Station |
| Risk | Medium - AWS service dependency |

# 4. Platform Requirements

This section specifies 14 functional requirements for platform-wide capabilities including authentication, authorization, and system operations.

## 4.1 Authentication & Authorization (FR-PLT-001 to FR-PLT-005)

### FR-PLT-001: User Authentication

| Requirement ID | FR-PLT-001 |
| --- | --- |
| Title | User Authentication |
| Description | The system shall authenticate users using username/password, SSO (OIDC), or certificate-based authentication. |
| Rationale | Secure authentication protects system access. |
| Source Use Case(s) | UC-SYS-001 (Authenticate User) |
| Priority | Critical |
| Acceptance Criteria | AC1: Support local username/password. AC2: Support OIDC SSO providers. AC3: Support X.509 certificates. AC4: Auth latency <500ms. AC5: Account lockout after failed attempts. |
| Dependencies | External: Identity Provider |
| Risk | Low |

### FR-PLT-002: Multi-Factor Authentication

| Requirement ID | FR-PLT-002 |
| --- | --- |
| Title | Multi-Factor Authentication |
| Description | The system shall support multi-factor authentication using TOTP, SMS, or hardware tokens for accounts with elevated privileges. |
| Rationale | MFA provides additional security layer for sensitive operations. |
| Source Use Case(s) | UC-SYS-001 (Authenticate User) |
| Priority | High |
| Acceptance Criteria | AC1: Support TOTP (Google Authenticator compatible). AC2: Support SMS codes. AC3: Support WebAuthn/FIDO2. AC4: Configurable MFA requirements by role. |
| Dependencies | FR-PLT-001 (Authentication) |
| Risk | Low |

### FR-PLT-003: Role-Based Access Control

| Requirement ID | FR-PLT-003 |
| --- | --- |
| Title | Role-Based Access Control |
| Description | The system shall enforce role-based access control with predefined roles (Admin, Operator, Analyst, Viewer) and custom role support. |
| Rationale | RBAC ensures users can only access authorized functions. |
| Source Use Case(s) | UC-SYS-002 (Authorize Action) |
| Priority | Critical |
| Acceptance Criteria | AC1: Predefined system roles with permissions. AC2: Support custom role creation. AC3: Role assignment by administrator. AC4: Permission inheritance. AC5: Decision latency <10ms. |
| Dependencies | FR-PLT-001 (Authentication) |
| Risk | Low |

### FR-PLT-004: Multi-Tenant Isolation

| Requirement ID | FR-PLT-004 |
| --- | --- |
| Title | Multi-Tenant Isolation |
| Description | The system shall provide complete data isolation between tenants with tenant-scoped access controls and resource limits. |
| Rationale | Multi-tenancy enables serving multiple customers on shared infrastructure. |
| Source Use Case(s) | UC-SYS-004 (Onboard Tenant), UC-SYS-005 (Configure Policies) |
| Priority | Critical |
| Acceptance Criteria | AC1: Complete data isolation between tenants. AC2: Tenant-scoped permissions. AC3: Resource quotas per tenant. AC4: No cross-tenant data leakage. |
| Dependencies | FR-PLT-003 (RBAC) |
| Risk | High - Security critical |

### FR-PLT-005: Audit Logging

| Requirement ID | FR-PLT-005 |
| --- | --- |
| Title | Audit Logging |
| Description | The system shall maintain comprehensive audit logs of all user actions, system events, and data access with tamper-evident storage. |
| Rationale | Audit logs support security investigation and compliance. |
| Source Use Case(s) | UC-SYS-007 (View Audit Logs) |
| Priority | High |
| Acceptance Criteria | AC1: Log all authentication events. AC2: Log all data access. AC3: Log all configuration changes. AC4: Tamper-evident storage. AC5: Retention per compliance requirements. |
| Dependencies | FR-PLT-001 (Authentication) |
| Risk | Low |

## 4.2 User & Tenant Management (FR-PLT-010 to FR-PLT-011)

### FR-PLT-010: User Account Management

| Requirement ID | FR-PLT-010 |
| --- | --- |
| Title | User Account Management |
| Description | The system shall support user account creation, modification, deactivation, and password management by administrators. |
| Rationale | User management enables adding and removing system access. |
| Source Use Case(s) | UC-SYS-003 (Manage User Account) |
| Priority | High |
| Acceptance Criteria | AC1: CRUD operations for users. AC2: Password reset capability. AC3: Account deactivation (not deletion). AC4: Bulk import from CSV. AC5: Support 10,000+ users. |
| Dependencies | FR-PLT-001 (Authentication), FR-PLT-003 (RBAC) |
| Risk | Low |

### FR-PLT-011: Tenant Provisioning

| Requirement ID | FR-PLT-011 |
| --- | --- |
| Title | Tenant Provisioning |
| Description | The system shall provision new tenants with isolated resources, initial administrator account, and configured entitlements. |
| Rationale | Tenant provisioning enables onboarding new customers. |
| Source Use Case(s) | UC-SYS-004 (Onboard Tenant) |
| Priority | High |
| Acceptance Criteria | AC1: Create tenant in <1 hour. AC2: Provision isolated database/storage. AC3: Create tenant admin account. AC4: Configure feature entitlements. AC5: Send welcome notification. |
| Dependencies | FR-PLT-004 (Multi-Tenant Isolation) |
| Risk | Medium |

## 4.3 Notifications (FR-PLT-020)

### FR-PLT-020: Notification Service

| Requirement ID | FR-PLT-020 |
| --- | --- |
| Title | Notification Service |
| Description | The system shall send notifications via multiple channels (email, SMS, Slack, webhooks) with templating and delivery tracking. |
| Rationale | Notifications keep stakeholders informed of events. |
| Source Use Case(s) | UC-GS-041 (Receive Alert), UC-SYS-009 (Configure Notifications) |
| Priority | High |
| Acceptance Criteria | AC1: Support email, SMS, Slack, webhook channels. AC2: Support notification templates. AC3: Track delivery status. AC4: User notification preferences. AC5: Delivery latency <30 seconds. |
| Dependencies | External: Email/SMS providers |
| Risk | Low |

## 4.4 System Operations (FR-PLT-030 to FR-PLT-031)

### FR-PLT-030: System Health Monitoring

| Requirement ID | FR-PLT-030 |
| --- | --- |
| Title | System Health Monitoring |
| Description | The system shall monitor health of all services and infrastructure with dashboards and alerting for degradations. |
| Rationale | Health monitoring enables proactive issue detection. |
| Source Use Case(s) | UC-SYS-006 (Monitor System Health) |
| Priority | Critical |
| Acceptance Criteria | AC1: Monitor all services (up/down/degraded). AC2: Track resource utilization (CPU, memory, disk). AC3: Dashboard with status visualization. AC4: Alert on threshold violations. AC5: Metric latency <30 seconds. |
| Dependencies | External: Monitoring stack (Prometheus/Grafana) |
| Risk | Low |

### FR-PLT-031: Backup and Recovery

| Requirement ID | FR-PLT-031 |
| --- | --- |
| Title | Backup and Recovery |
| Description | The system shall perform automated backups of all critical data with tested recovery procedures. |
| Rationale | Backups protect against data loss. |
| Source Use Case(s) | UC-SYS-010 (Backup System Data) |
| Priority | High |
| Acceptance Criteria | AC1: Daily automated backups. AC2: Configurable retention (default 30 days). AC3: Backup verification. AC4: Recovery time <4 hours. AC5: Point-in-time recovery capability. |
| Dependencies | External: Backup storage |
| Risk | Medium |

## 4.5 API Management (FR-PLT-040 to FR-PLT-041)

### FR-PLT-040: API Authentication

| Requirement ID | FR-PLT-040 |
| --- | --- |
| Title | API Authentication |
| Description | The system shall authenticate API requests using API keys or OAuth2 tokens with rate limiting. |
| Rationale | API authentication enables secure programmatic access. |
| Source Use Case(s) | UC-SYS-008 (Manage API Keys) |
| Priority | High |
| Acceptance Criteria | AC1: Support API key authentication. AC2: Support OAuth2 bearer tokens. AC3: Rate limiting per key/user. AC4: API key rotation. AC5: Key usage tracking. |
| Dependencies | FR-PLT-001 (Authentication) |
| Risk | Low |

### FR-PLT-041: API Documentation

| Requirement ID | FR-PLT-041 |
| --- | --- |
| Title | API Documentation |
| Description | The system shall provide interactive API documentation with try-it-out capability for all public endpoints. |
| Rationale | Documentation enables API integration by developers. |
| Source Use Case(s) | UC-API-001 to UC-API-004 (API Developer use cases) |
| Priority | Medium |
| Acceptance Criteria | AC1: OpenAPI 3.0 specification. AC2: Interactive Swagger UI. AC3: Code examples in multiple languages. AC4: Auto-generated from code. |
| Dependencies | All API endpoints |
| Risk | Low |

# 5. Continuation

This document continues in Part 2 with:

- Section 6: Earth Observation Analytics Requirements (35 requirements)

- Section 7: Small Satellite Subsystems Requirements (30 requirements)

- Section 8: Geospatial Intelligence Requirements (25 requirements)

- Section 9: Non-Functional Requirements (20 requirements)

- Appendix A: Requirements Traceability Matrix