P9E MICROSYSTEMS PVT LTD

Chetana Space Technologies Division

# Use Case Diagrams

Space Technology Platform

Use Case Modeling - Phase 2, Deliverable D2.2

| Document ID: | P9E-SPACE-UC-D2.2-001 |
| --- | --- |
| Version: | 1.0 |
| Date: | December 27, 2025 |
| Total Use Cases: | 103 Use Cases across 5 Modules |

# Table of Contents

# 1. Executive Summary

This document presents the Use Case Diagrams for the P9e Space Technology Platform. It identifies 103 use cases across 5 modules, providing a comprehensive view of system functionality and actor interactions.

## 1.1 Use Case Statistics

| Module | Use Cases | Critical | Primary Actors |
| --- | --- | --- | --- |
| Ground Station Software | 23 | 6 | Mission Operator, GS Engineer |
| Earth Observation Analytics | 24 | 0 | EO Analyst, Data Scientist |
| Small Satellite Subsystems | 26 | 8 | Sat Sys Engineer, Payload Op |
| Geospatial Intelligence | 20 | 0 | GEOINT Analyst |
| Platform / Administration | 10 | 2 | Platform Admin, Sys Admin |
| TOTAL | 103 | 16 |  |

## 1.2 How to Read Use Case Diagrams

Each module section contains:

- Use Case Diagram: Visual representation in PlantUML/Mermaid format (render using online tools)

- Use Case Table: Complete listing with ID, name, primary actor, and priority

- Actor Mapping: Which actors participate in which use cases

- Category Grouping: Use cases organized by functional area

# 2. Ground Station Software Module

The Ground Station Software module handles all mission control operations including satellite pass execution, telemetry monitoring, command uplink, and antenna control. This is the most operationally critical module with real-time requirements.

## 2.1 Ground Station Use Case Diagram (PlantUML)

Render this diagram at plantuml.com or using VS Code PlantUML extension:

@startuml Ground_Station_Use_Cases

left to right direction

skinparam packageStyle rectangle

actor "Mission Operator" as MO

actor "GS Engineer" as GSE

actor "Sat Sys Engineer" as SSE

actor "Satellite" as SAT <<system>>

actor "Antenna System" as ANT <<system>>

actor "Space-Track" as ST <<system>>

actor "AWS Ground Station" as AWS <<system>>

rectangle "Ground Station Software" {

package "Pass Operations" {

(UC-GS-001 Schedule Pass) as UC001

(UC-GS-002 Execute Pass) as UC002

(UC-GS-003 Monitor Telemetry) as UC003

(UC-GS-004 Send Command) as UC004

(UC-GS-005 Handle Anomaly) as UC005

}

package "Antenna & RF" {

(UC-GS-010 Configure RF) as UC010

(UC-GS-011 Calibrate Antenna) as UC011

(UC-GS-013 Control Pointing) as UC013

}

package "Scheduling" {

(UC-GS-020 Predict Passes) as UC020

(UC-GS-021 Resolve Conflicts) as UC021

(UC-GS-030 Update TLEs) as UC030

}

}

MO --> UC001

MO --> UC002

MO --> UC003

MO --> UC004

MO --> UC005

MO --> UC021

GSE --> UC010

GSE --> UC011

SSE --> UC003

UC002 --> SAT

UC013 --> ANT

ST --> UC030

UC002 ..> UC013 : <<include>>

UC002 ..> UC003 : <<include>>

UC001 ..> UC020 : <<include>>

@enduml

## 2.2 Ground Station Use Case List

| UC ID | Use Case Name | Primary Actor | Priority |
| --- | --- | --- | --- |
| UC-GS-001 | Schedule Satellite Pass | Mission Operator | High |
| UC-GS-002 | Execute Satellite Pass | Mission Operator | Critical |
| UC-GS-003 | Monitor Real-Time Telemetry | Mission Operator | Critical |
| UC-GS-004 | Send Telecommand | Mission Operator | Critical |
| UC-GS-005 | Handle Pass Anomaly | Mission Operator | High |
| UC-GS-006 | Record Pass Data | System (Automatic) | Critical |
| UC-GS-007 | Generate Pass Report | Mission Operator | Medium |
| UC-GS-010 | Configure RF Equipment | GS Engineer | High |
| UC-GS-011 | Calibrate Antenna | GS Engineer | Medium |
| UC-GS-012 | Run Station Diagnostics | GS Engineer | Medium |
| UC-GS-013 | Control Antenna Pointing | System (Automatic) | Critical |
| UC-GS-014 | Monitor Signal Quality | Mission Operator | High |
| UC-GS-020 | Predict Pass Windows | System (Automatic) | High |
| UC-GS-021 | Resolve Schedule Conflicts | Mission Operator | High |
| UC-GS-022 | Coordinate Multi-Station | Mission Operator | Medium |
| UC-GS-023 | Integrate External GS Network | System (AWS GS) | Medium |
| UC-GS-030 | Update TLE Database | System (Space-Track) | High |
| UC-GS-031 | Archive Telemetry Data | System (Automatic) | High |
| UC-GS-032 | Retrieve Historical Telemetry | Satellite Sys Engineer | Medium |
| UC-GS-033 | Export Pass Data | Mission Operator | Low |
| UC-GS-040 | Configure Alert Thresholds | Mission Operator | High |
| UC-GS-041 | Receive Anomaly Alert | Mission Operator | Critical |
| UC-GS-042 | Acknowledge Alert | Mission Operator | High |

## 2.3 Ground Station Use Case Details

### UC-GS-002: Execute Satellite Pass

| Use Case ID | UC-GS-002 |
| --- | --- |
| Name | Execute Satellite Pass |
| Primary Actor | Mission Operator (ACT-H-001) |
| Secondary Actors | Satellite Spacecraft (ACT-S-001), Antenna Control System (ACT-S-002), SDR (ACT-S-003) |
| Priority | CRITICAL - Core mission operation |
| Description | Mission Operator executes a scheduled satellite pass, controlling antenna tracking, establishing RF link, receiving telemetry, and sending commands during the pass window. |
| Preconditions | 1) Pass is scheduled (UC-GS-001). 2) TLEs are current (<24 hrs). 3) Ground station is operational. 4) RF equipment is configured. 5) Operator is authenticated. |
| Main Flow | 1) System alerts operator 10 min before AOS. 2) Operator reviews pass plan. 3) System initiates antenna pre-positioning at T-5 min. 4) At AOS, antenna begins tracking. 5) SDR acquires signal and locks. 6) Telemetry stream begins (include UC-GS-003). 7) Operator sends commands as needed (include UC-GS-004). 8) System records all data (include UC-GS-006). 9) At LOS, system stows antenna. 10) System generates pass summary. |
| Alternate Flows | A1) Signal not acquired: System retries autotrack, falls back to program track. A2) Telemetry errors: System logs errors, continues recording. A3) Command failed: System retries, alerts operator. |
| Exception Flows | E1) Antenna failure: Emergency stow, abort pass, trigger UC-GS-005. E2) Loss of spacecraft: Initiate contingency search. E3) Weather abort: Cancel pass, reschedule. |
| Postconditions | 1) All telemetry recorded and archived. 2) Commands logged with acknowledgments. 3) Pass report generated. 4) Antenna in safe position. 5) Data pipeline triggered. |
| Includes | UC-GS-003 (Monitor Telemetry), UC-GS-006 (Record Data), UC-GS-013 (Control Pointing) |
| Extended By | UC-GS-004 (Send Command), UC-GS-005 (Handle Anomaly) |
| Business Rules | BR-1: TLEs must be <24 hours old. BR-2: Minimum elevation 5°. BR-3: Two-person rule for critical commands. BR-4: All passes must be logged. |

# 3. Earth Observation Analytics Module

The Earth Observation Analytics module handles satellite imagery ingestion, processing, analysis, and product generation. It includes ML pipelines for automated analysis.

## 3.1 EO Analytics Use Case Diagram (Mermaid)

Render this diagram at mermaid.live:

flowchart LR

subgraph Actors

EOA["👤 EO Analyst"]

DS["👤 Data Scientist"]

COP["🖥️ Copernicus"]

PLN["🖥️ Planet API"]

end

subgraph EO["Earth Observation Analytics"]

UC001((Search Catalog))

UC010((Process Image))

UC020((Classify Land Cover))

UC021((Detect Objects))

UC022((Detect Changes))

UC030((Train Model))

UC032((Deploy Model))

UC040((Export Products))

end

EOA --> UC001 & UC010 & UC020 & UC021 & UC022 & UC040

DS --> UC030 & UC032

COP & PLN --> UC001

## 3.2 EO Analytics Use Case List

| UC ID | Use Case Name | Primary Actor | Priority |
| --- | --- | --- | --- |
| UC-EO-001 | Search Imagery Catalog | EO Analyst | High |
| UC-EO-002 | Ingest Sentinel Data | System (Copernicus) | High |
| UC-EO-003 | Ingest Landsat Data | System (USGS) | High |
| UC-EO-004 | Ingest Commercial Data | System (Planet/Maxar) | Medium |
| UC-EO-005 | Validate Data Quality | System (Automatic) | High |
| UC-EO-010 | Apply Radiometric Correction | EO Analyst | High |
| UC-EO-011 | Apply Geometric Correction | EO Analyst | High |
| UC-EO-012 | Apply Atmospheric Correction | EO Analyst | High |
| UC-EO-013 | Create Image Mosaic | EO Analyst | Medium |
| UC-EO-014 | Process SAR Data | EO Analyst | High |
| UC-EO-015 | Run Processing Pipeline | EO Analyst | High |
| UC-EO-020 | Classify Land Cover | EO Analyst | High |
| UC-EO-021 | Detect Objects | EO Analyst | High |
| UC-EO-022 | Detect Changes | EO Analyst | High |
| UC-EO-023 | Calculate Vegetation Indices | EO Analyst | Medium |
| UC-EO-024 | Extract Building Footprints | EO Analyst | Medium |
| UC-EO-025 | Validate Analysis Results | EO Analyst | High |
| UC-EO-030 | Train ML Model | Data Scientist | High |
| UC-EO-031 | Evaluate Model Performance | Data Scientist | High |
| UC-EO-032 | Deploy Model to Production | Data Scientist | High |
| UC-EO-033 | Monitor Model Drift | Data Scientist | Medium |
| UC-EO-034 | Manage Training Data | Data Scientist | Medium |
| UC-EO-040 | Export Analysis Products | EO Analyst | High |
| UC-EO-041 | Generate Quicklook | System (Automatic) | Medium |
| UC-EO-042 | Create Time Series | EO Analyst | Medium |

# 4. Small Satellite Subsystems Module

The Small Satellite Subsystems module provides flight software frameworks, subsystem monitoring, and simulation capabilities for CubeSat/SmallSat missions.

## 4.1 Satellite Subsystems Use Case List

| UC ID | Use Case Name | Primary Actor | Priority |
| --- | --- | --- | --- |
| UC-SAT-001 | Estimate Attitude | System (Automatic) | Critical |
| UC-SAT-002 | Control Attitude | System (Automatic) | Critical |
| UC-SAT-003 | Execute Pointing Mode | Payload Operator | High |
| UC-SAT-004 | Calibrate Sensors | Satellite Sys Engineer | Medium |
| UC-SAT-010 | Process Telecommand | System (Automatic) | Critical |
| UC-SAT-011 | Generate Telemetry | System (Automatic) | Critical |
| UC-SAT-012 | Schedule Onboard Task | Mission Operator | High |
| UC-SAT-013 | Manage Onboard Storage | System (Automatic) | High |
| UC-SAT-014 | Execute Onboard Script | System (Automatic) | Medium |
| UC-SAT-020 | Monitor Power State | Satellite Sys Engineer | Critical |
| UC-SAT-021 | Manage Battery | System (Automatic) | Critical |
| UC-SAT-022 | Control Load Shedding | System (Automatic) | High |
| UC-SAT-023 | Predict Eclipse Power | Flight Dynamics Eng | Medium |
| UC-SAT-030 | Monitor Thermal State | Satellite Sys Engineer | High |
| UC-SAT-031 | Control Heaters | System (Automatic) | High |
| UC-SAT-040 | Plan Collection | Payload Operator | High |
| UC-SAT-041 | Task Payload | Payload Operator | High |
| UC-SAT-042 | Calibrate Payload | Payload Operator | Medium |
| UC-SAT-043 | Download Payload Data | Mission Operator | Critical |
| UC-SAT-050 | Determine Orbit | Flight Dynamics Eng | High |
| UC-SAT-051 | Plan Maneuver | Flight Dynamics Eng | High |
| UC-SAT-052 | Assess Conjunction | Flight Dynamics Eng | Critical |
| UC-SAT-053 | Propagate Orbit | System (Automatic) | High |
| UC-SAT-060 | Run SITL Simulation | Satellite Sys Engineer | High |
| UC-SAT-061 | Execute HITL Test | Satellite Sys Engineer | Medium |
| UC-SAT-062 | Perform Monte Carlo Analysis | Flight Dynamics Eng | Medium |

## 4.2 Satellite Subsystems Use Case Details

### UC-SAT-052: Assess Conjunction Risk

| Use Case ID | UC-SAT-052 |
| --- | --- |
| Name | Assess Conjunction Risk |
| Primary Actor | Flight Dynamics Engineer (ACT-H-006) |
| Secondary Actors | Space-Track.org (ACT-S-009), 18th Space Control Squadron CDMs |
| Priority | CRITICAL - Spacecraft safety |
| Description | Flight Dynamics Engineer assesses collision risk from Conjunction Data Messages (CDMs) and determines if a collision avoidance maneuver is required. |
| Preconditions | 1) CDM received or screening performed. 2) Current ephemeris available. 3) Satellite propulsion status known. |
| Main Flow | 1) System receives CDM from Space-Track. 2) System calculates probability of collision (Pc). 3) If Pc > threshold, alert FD Engineer. 4) Engineer reviews conjunction geometry. 5) Engineer runs Monte Carlo analysis. 6) If maneuver needed, plan CAM (UC-SAT-051). 7) Document decision and notify stakeholders. |
| Postconditions | 1) Conjunction risk documented. 2) Maneuver decision recorded. 3) If CAM planned, UC-SAT-051 initiated. 4) Stakeholders notified. |

# 5. Geospatial Intelligence Module

The Geospatial Intelligence module transforms raw spatial data into actionable intelligence through multi-source fusion, pattern analysis, and automated reporting.

## 5.1 GEOINT Use Case List

| UC ID | Use Case Name | Primary Actor | Priority |
| --- | --- | --- | --- |
| UC-GI-001 | Create Area of Interest | GEOINT Analyst | High |
| UC-GI-002 | Monitor AOI | GEOINT Analyst | High |
| UC-GI-003 | Share AOI with Team | GEOINT Analyst | Medium |
| UC-GI-010 | Fuse Multi-Source Data | GEOINT Analyst | High |
| UC-GI-011 | Integrate Ground Truth | GEOINT Analyst | Medium |
| UC-GI-012 | Load Reference Data | System (OSM) | Medium |
| UC-GI-013 | Stack Temporal Data | GEOINT Analyst | High |
| UC-GI-020 | Analyze Activity Patterns | GEOINT Analyst | High |
| UC-GI-021 | Detect Anomalies | GEOINT Analyst | High |
| UC-GI-022 | Track Infrastructure Changes | GEOINT Analyst | High |
| UC-GI-023 | Monitor Maritime Activity | GEOINT Analyst | Medium |
| UC-GI-024 | Assess Environmental Impact | GEOINT Analyst | Medium |
| UC-GI-030 | View Multi-Layer Map | GEOINT Analyst | High |
| UC-GI-031 | Visualize 3D Terrain | GEOINT Analyst | Medium |
| UC-GI-032 | Annotate Imagery | GEOINT Analyst | High |
| UC-GI-033 | Use Temporal Slider | GEOINT Analyst | Medium |
| UC-GI-040 | Generate Intelligence Report | GEOINT Analyst | High |
| UC-GI-041 | Export Products | GEOINT Analyst | High |
| UC-GI-042 | Schedule Automated Report | GEOINT Analyst | Medium |
| UC-GI-050 | Share Analysis Workspace | GEOINT Analyst | Medium |
| UC-GI-051 | Comment on Analysis | GEOINT Analyst | Medium |
| UC-GI-052 | Review Peer Analysis | GEOINT Analyst | Medium |

# 6. Platform / Administration Module

The Platform module handles cross-cutting concerns including authentication, authorization, tenant management, and system administration.

## 6.1 Platform Use Case List

| UC ID | Use Case Name | Primary Actor | Priority |
| --- | --- | --- | --- |
| UC-SYS-001 | Authenticate User | All Users | Critical |
| UC-SYS-002 | Authorize Action | System (Chetana) | Critical |
| UC-SYS-003 | Manage User Account | Platform Admin | High |
| UC-SYS-004 | Onboard Tenant | Platform Admin | High |
| UC-SYS-005 | Configure Tenant Policies | Platform Admin | High |
| UC-SYS-006 | Monitor System Health | System Admin | Critical |
| UC-SYS-007 | View Audit Logs | Security Officer | High |
| UC-SYS-008 | Manage API Keys | API Developer | High |
| UC-SYS-009 | Configure Notifications | All Users | Medium |
| UC-SYS-010 | Backup System Data | System Admin | High |

# 7. Use Case to Actor Traceability

This section maps actors to their use cases for traceability.

## 7.1 Primary Actor Use Case Count

| Actor | Use Cases | Primary Module |
| --- | --- | --- |
| Mission Operator | 15 | Ground Station Software |
| EO Analyst | 18 | Earth Observation Analytics |
| GEOINT Analyst | 20 | Geospatial Intelligence |
| Data Scientist | 5 | Earth Observation Analytics (ML) |
| Satellite Systems Engineer | 8 | Small Satellite Subsystems |
| Flight Dynamics Engineer | 6 | Small Satellite Subsystems (Orbit) |
| Ground Station Engineer | 4 | Ground Station Software |
| Payload Operator | 4 | Small Satellite Subsystems (Payload) |

# 8. Next Steps

With Use Case Diagrams (D2.2) complete, the next deliverable is:

- D2.3 Use Case Specifications: Detailed specification for each of the 103 use cases (prioritized by Critical/High first)

- D3.1 Requirements Inventory: Derive detailed requirements from use cases

- D3.4 Requirements Traceability Matrix: Map Use Cases → Requirements